CYBER SECURITY CLUB  |  CONFIDENTIAL TRAINING FILE

Introduction to Wireshark

Network Analyst Report  •  Case #001: “Who Is My Computer Talking To?”

| Analyst Name | Date | Case Status |
| --- | --- | --- |
|  |  | OPEN |

Mission briefing: Every time you open a website, your computer sends and receives hundreds of tiny messages called packets. Wireshark lets you see them. In this case file you will capture your own traffic, find out who your computer asked for directions, uncover your own IP address, and read the “doors” (ports) your computer used to talk to the internet. Fill in every answer box. Your report is your evidence.

Rules of the lab: Only capture on your own workstation, and only inside this lab. Analysts never share what they find without permission.

PART 1 – GENERATE NETWORK TRAFFIC

- Open Wireshark. On the home screen, click the interface that has a moving line graph next to it (usually Wi-Fi or Ethernet).

- Click the Start Capture (blue shark fin) button.

- Open a web browser and visit two or three websites of your choice (school-appropriate!). Stay about 20 seconds.

- Return to Wireshark and click the Stop (red square) button.

Q1. Which interface did you capture on, and how many packets did you capture? (Look at the bottom status bar: “Packets: ___”)

Your answer:

Q2. Name the websites you visited while capturing.

Your answer:

Decode the packet. Click any packet in the list. In the Packet Details pane (middle), you will see about five lines. Each line is one layer of the OSI model, starting at the bottom of the stack.

Q3. Complete the table. Use the word bank to fill in the OSI layer number and name for each line.

Word bank: Physical (1)  •  Data Link (2)  •  Network (3)  •  Transport (4)  •  Application (7)

| Line in Wireshark | What it handles | Layer # | Layer name |
| --- | --- | --- | --- |
| Frame | The raw bits on the wire |  |  |
| Ethernet II | MAC addresses (device to device) |  |  |
| Internet Protocol Version 4 | IP addresses (computer to computer) |  |  |
| TCP or UDP | Ports (program to program) |  |  |
| DNS, TLS, or HTTP | The actual app data |  |  |

PART 2 – VIEW DNS TRAFFIC

Before your computer can visit a website, it has to ask: “What is the address for this name?” That question is a DNS query.

- Click inside the Display Filter bar (the green bar at the top).

- Type: dns

- Press Enter.

- Select any DNS packet whose Info column says Standard query.

- In the Packet Details pane, expand:

  - Domain Name System

  - Queries

- Identify one website your computer requested.

Q4. Which website (domain name) did your computer ask about?

Your answer:

Q5. What is DNS and why does your computer need it? (Hint: think of a phone contacts list.)

Your answer:

Q6. Now find the answer. Look for a packet that says Standard query response for the same website. Expand Domain Name System > Answers. What IP address did DNS give back?

Your answer:

Q7. Paste a screenshot of your expanded Queries section below (press Win + Shift + S to snip).

EVIDENCE EXHIBIT Paste your screenshot here

PART 3 – FIND YOUR IP ADDRESS

- Make sure your Display Filter still says dns and press Enter.

- Find a packet in the Info column that says Standard query.

- In the Packet Details pane, expand:

  - Internet Protocol Version 4

- Locate the Source Address field. That is your computer.

- Now locate the Destination Address field. That is who you asked.

Q8. What is your computer’s IP address?

Your answer:

Q9. What is the Destination IP address of your DNS query? (This is usually your router or the school’s DNS server.)

Your answer:

Q10. Private IP addresses start with 10.  or 172.16–31.  or 192.168.  Is your IP address private or public?

Your answer:

PART 4 – VIEW TCP TRAFFIC

- Clear the Display Filter (click the X at the right end of the bar).

- Type: tcp

- Press Enter.

- Select any TCP packet.

- In the Packet Details pane, expand Transmission Control Protocol, then locate:

  - Source Port

  - Destination Port

Port cheat sheet (a port is like an apartment number: the IP is the building, the port is which door)

| Port | Service |
| --- | --- |
| 22 | SSH (secure remote login) |
| 53 | DNS |
| 80 | HTTP (web, not encrypted) |
| 443 | HTTPS (web, encrypted) |

Q11. What are the Source Port and Destination Port of your TCP packet?

Your answer:

Q12. Look at your two port numbers. Which one is on the cheat sheet, and what service is it? (If neither is, write "not on the list." The other number is just a random one your computer picked.)

Your answer:

Q13. Challenge: Type  tcp.port == 443  in the Display Filter and press Enter. Click any packet and look at the Protocol column. What does it say? Can you read what is inside the packet?

Your answer:

PART 5 – EVIDENCE SUMMARY & SIGN-OFF

Copy your key findings from this case into the evidence log below.

| Evidence | Finding |
| --- | --- |
| Website I visited |  |
| My IP address |  |
| DNS server IP |  |
| IP address of the website |  |
| Source port / Destination port |  |

Q14. Analyst conclusion: In 3–4 sentences, explain what we did and what commands we used. Use some terms from this lab (DNS, IP address, port, packet, TCP).

Your answer:

| ANALYST SIGNATURE |  | DATE |  |
| --- | --- | --- | --- |

CASE #001 CLOSED  •  RANK EARNED: JUNIOR PACKET ANALYST

PART 6 – EXTRA: CATCH YOUR COMPUTER JOINING THE NETWORK

Finished early? This part is extra credit. Do it if time permits and add it to your report.

When your computer joins a network, it does not have an IP address yet. It has to ask for one. In this part you will catch that conversation as it happens.

- Click the Start Capture button. (If Wireshark asks to save your old capture, click Continue without Saving.)

- Disconnect from Wi-Fi: click the Wi-Fi icon in the taskbar and choose Disconnect. (On Ethernet, unplug the cable.) Wait about 5 seconds.

- Reconnect to Wi-Fi (or plug the cable back in). Wait until the internet works again.

- Click the Stop button.

- In the Display Filter, type dhcp and press Enter. (If nothing shows up, try bootp instead.)

- Look at the Info column. You should see a DHCP Request and a DHCP ACK. (Some computers show two more packets, Discover and Offer. That is normal too.)

Q15. Look at your two packets. Which packet number is the DHCP Request, and which is the DHCP ACK? (Hint: the Request happens first, so it has the lower number.)

DHCP Request = packet # DHCP ACK = packet #

Q16. Click the Request packet. What are its Source and Destination IP addresses? (Hint: your computer is shouting to everyone on the network.)

Your answer:

Q17. Click the ACK packet. In the Packet Details pane, expand Dynamic Host Configuration Protocol and find “Your (client) IP address.” What IP address did the router give you? Is it the same as your answer to Q8?

Your answer:

Q18. Scroll down in the same section. Find Option: (3) Router and Option: (51) IP Address Lease Time. What is the router’s IP address, and how long is your lease (how long you get to keep the address)?

Your answer:

BONUS EVIDENCE LOGGED  •  EXTRA CREDIT EARNED